6-DOF Robot Arm Vision Pipeline

Pick & Drop — Small Mixed Parts (Screws, Electronics, Tools)

# 1. Vision Representation Strategy

Use different 3D representations for different pipeline stages — not one format for everything.

| Representation | Role | Notes |
| --- | --- | --- |
| RGB + Depth (raw) | Input layer only | Feed RGB to YOLO; sample depth inside bbox |
| Point Cloud | Core — pose estimation | Native output from sensor; use for all 6D work |
| Voxel Grid | Collision / motion planning | Feed into MoveIt; too coarse for small parts |
| Mesh (CAD) | Offline reference models | Match against live point cloud; don't generate live |
| Implicit / TSDF | Skip | Only useful for multi-view fusion; too heavy |

# 2. Depth-First Filtering Pipeline (Before YOLO)

Use depth to reduce the YOLO search area and identify the most pickable candidates first.

## Stage 1 — Depth Pre-filter

- Run RANSAC plane removal on the depth map to eliminate the bin floor

- Slice the top 15–20 mm in world-frame Z to get the candidate zone

- Apply morphological cleanup to remove noise pixels

- Project the resulting mask onto the RGB image — pass only this region to YOLO

## Stage 2 — World-Frame Projection (Robot Kinematics)

Because the camera is wrist-mounted, the depth Z-axis is the optical axis, not true vertical. Correct for this:

- Compute T_world_to_camera = T_base_to_ee × T_ee_to_camera (one-time hand-eye calibration via cv2.calibrateHandEye)

- Project every depth pixel into world coordinates to get true height above the bin floor

- The gravity vector in camera space is now known — use it to compute surface normal alignment

- RealSense IMU can provide a second independent gravity estimate as a sanity check

## Stage 3 — Occlusion Check (per YOLO detection)

- Depth coherence: check that depth values inside the bounding box are uniform (one object, not a stack)

- Depth gradient at bbox edges: sharp discontinuity on one edge = something is on top

- Visible surface ratio: compare detected top-surface area vs expected area from CAD model — if < 70%, skip

## Stage 4 — Pickability Score

Score each detection before committing to a grasp:

| Term | What It Measures |
| --- | --- |
| world_height | True vertical height above bin floor (highest = most accessible) |
| visible_surface_ratio | Fraction of expected top surface that is unoccluded |
| normal_alignment_with_gravity | Dot product of surface normal with gravity vector (1.0 = flat/horizontal) |
| depth_coherence | Variance of depth values inside the bounding box (low = clean isolated object) |
| reachability_from_current_pose | IK feasibility check — can the arm reach this pose without singularity |

score = w1×world_height + w2×visible_ratio + w3×normal_alignment + w4×depth_coherence + w5×reachability

# 3. Grasp Pose Estimation — Parallel Jaw Gripper

Output required: position (x,y,z) + approach vector + jaw axis + jaw opening width.

## Step 1 — Fit a Geometric Primitive (PCA / OBB)

- Crop the point cloud to the detected object's bounding box

- Run PCA on the cropped cloud — eigenvectors give the 3 principal object axes

- Fit an Oriented Bounding Box (OBB); longest axis = main orientation, shortest = thickness

- For screws/cylindrical tools: fit a cylinder instead of OBB

## Step 2 — Determine Jaw Axis (Angle)

- Jaw axis = longest OBB axis projected onto the horizontal plane

- Fingers close along the SHORT dimension of the object for stable contact

- 180° ambiguity: pick the approach direction that avoids bin walls (check voxel grid)

## Step 3 — Antipodal Point Search

Find the best pair of contact points (P, Q) for the two fingers:

- For each surface point P: cast ray through object center, find exit point Q

- Valid pair conditions: distance(P,Q) < jaw_max_width, normals at P and Q roughly opposing, approach path clear

- Score all valid pairs; highest score = grasp candidate

## Step 4 — Grasp Quality Score

| Term | What It Measures |
| --- | --- |
| antipodal_normal_alignment | Fingers grip, not slip — normals oppose each other |
| approach_clearance | No obstacles along the descent path (checked against voxel grid) |
| distance_from_centroid | Stable grasp near object center of mass |
| jaw_width_margin | Not near maximum jaw opening — leaves tolerance |
| wrist_reachability | Full IK feasibility including wrist orientation |

# 4. Compute Stack & Recommended Libraries

| Task | Library / Tool | Notes |
| --- | --- | --- |
| Object detection | YOLOv8n or YOLOv10n | Run on masked RGB only; nano variant for Pi 5 |
| Depth processing | Open3D | RANSAC, PCA, OBB, point cloud ops |
| Pose estimation | FoundationPose / SAM-6D | Zero-shot; needs CAD mesh as reference |
| Grasp planning | GPD or custom OBB+antipodal | GPD is heavier; custom is Pi-friendly |
| Motion planning | MoveIt 2 (ROS 2) | Feed voxel grid as collision scene |
| Hand-eye calib. | cv2.calibrateHandEye | One-time setup; ~15 poses with checkerboard |
| Accelerator (opt.) | Hailo-8L HAT+ for Pi 5 | ~13 TOPS; runs YOLOv8s at 30 FPS on Pi 5 |

# 5. Full Pipeline Summary

| Stage | Input | Output |
| --- | --- | --- |
| 1. Depth pre-filter | Raw depth frame | Binary mask of top-of-stack pixels |
| 2. World projection | Mask + robot joint angles | True-height map; gravity vector in camera frame |
| 3. YOLO detection | Masked RGB region | Bounding boxes + class labels |
| 4. Occlusion check | BBoxes + depth | Per-detection occlusion score |
| 5. Pickability score | All above signals | Ranked list of candidate objects |
| 6. Primitive fitting | Cropped point cloud | OBB / cylinder axes + dimensions |
| 7. Antipodal search | Point cloud + OBB | Candidate grasp pairs (P, Q) |
| 8. Grasp scoring | All grasp candidates | Best grasp pose (position, approach, jaw axis, width) |
| 9. Motion plan | Grasp pose + voxel grid | Collision-free trajectory to gripper |

Key insight: depth filters candidates → YOLO identifies objects → point cloud determines grasp